Documenting Iterative Development

A worked example: ESP32 LED project, Timer → Button → WiFi, in the V-Model

# 1. Approach: a V-Cycle per Feature

Rather than one giant V for the whole project, each new capability runs through its own small V — requirements, design, code, and test — and stacks on top of what came before. Two artifacts tie the cycles together: a change log (what changed, when, with what result) and a requirements traceability matrix (every requirement, mapped to the design, code, and test that satisfy it). Every cycle after the first must also regression-test what earlier cycles already proved, since new inputs can interfere with old ones — that's exactly what happens below when WiFi is added in V3.

# 2. Change Log

| Cycle | Date | Feature Added | What Changed | Test Result |
| --- | --- | --- | --- | --- |
| V1 | Day 1 | Timer-based blink | New: setup()/loop() timer logic; ledOn()/ledOff() | Pass — LED blinks every 1s reliably |
| V2 | Day 3 | Button override | New: readButton() with debounce; loop() now checks button before applying timer state | Pass — button overrides blink instantly; blink resumes correctly after release (timer regression pass) |
| V3 | Day 7 | WiFi toggle | New: minimal web server + handleToggle(); shared ledState variable now written from 3 sources | Pass — WiFi toggle works; button/timer regression pass; race condition found and fixed (see REQ-4) |

# 3. Requirements Traceability Matrix

This is the master record: every requirement across all three cycles, and proof it was designed, built, and tested.

| ID | Requirement | Cycle | Design Ref | Code Module | Test Case(s) | Status |
| --- | --- | --- | --- | --- | --- | --- |
| REQ-1 | LED shall blink automatically every 1s when idle | V1 | Sys. Design 1.1 | loop() timer branch | TC-1.1 Unit (interval) TC-1.2 System (50-cycle) | Pass |
| REQ-2 | Button press shall override blink and toggle LED immediately | V2 | Sys. Design 2.1 (state diagram) | readButton(), loop() priority logic | TC-2.1 Unit (debounce) TC-2.2 Integration TC-2.3 Regression (timer resumes) | Pass |
| REQ-3 | LED shall be toggle-able via a WiFi HTTP request | V3 | Architecture 3.1 (web server) | handleToggle(), WiFi setup | TC-3.1 Unit (endpoint) TC-3.2 Integration (HTTP→GPIO) TC-3.3 Regression (button/timer) | Pass |
| REQ-4 | LED state shall stay consistent regardless of which input changed it last | V3 (added mid-cycle, see note below) | Architecture 3.2 (shared state) | ledState global + critical section | TC-4.1 System (rapid alternating inputs) TC-4.2 Acceptance (no flicker/race) | Pass |

Note on REQ-4: it didn't exist in the original V1/V2 requirements — it surfaced during V3 integration testing when the WiFi handler and the button handler both wrote to the LED state without coordination, causing an intermittent race. This is normal: new requirements are allowed to enter the matrix mid-project as long as they get their own design/code/test trail, same as any other requirement. The matrix is a living document, not a one-time upfront list.

# 4. Cycle Detail

Each cycle below follows the same V shape as the original plan, scoped to just what's new.

## 4.1 V1 — Timer

Baseline cycle; no prior behavior to regress against.

Requirements added: REQ-1: LED shall blink automatically every 1s when idle.

Design delta: State diagram: OFF → (1s) → ON → (1s) → OFF, driven by millis() in loop().

Code delta: setup(), loop() with timer branch, ledOn(), ledOff().

Unit testing: TC-1.1: verify ledOn()/ledOff() toggle the physical pin via Serial Monitor prints.

Integration testing: Flash board, confirm GPIO write actually drives the external LED through the resistor.

System testing: TC-1.2: run 50 blink cycles uninterrupted, confirm no missed transitions or hangs.

Acceptance testing: Confirm blink rate is visibly ~1s and matches REQ-1.

Deliverable: Flashed sketch (esp32_led_v1.ino) implementing the timer blink, the V1 state diagram, and a 50-cycle test log confirming REQ-1.

## 4.2 V2 — Button Override

Builds on V1; must not break the timer behavior.

Requirements added: REQ-2: pressing the button shall override the blink and toggle the LED immediately.

Design delta: State diagram extended: button press forces an immediate state change and pauses the timer for a short window; state diagram updated to show button as a higher-priority input.

Code delta: readButton() (with ~30ms debounce), loop() restructured so button input is checked before the timer branch.

Unit testing: TC-2.1: verify readButton() reports a single clean press with no false triggers (debounce check).

Integration testing: TC-2.2: confirm a physical button press reliably toggles the real LED on the breadboard.

System / Regression testing: TC-2.3: after releasing the button, confirm the timer resumes blinking correctly — this is the regression check that V1's behavior wasn't broken by V2's change.

Acceptance testing: Confirm both behaviors coexist as expected: idle → blinks; press → overrides instantly.

Deliverable: Updated sketch (esp32_led_v2.ino) adding the debounced button override, the revised V2 state diagram showing button priority, and a regression log proving V1's timer behavior still passes alongside REQ-2.

## 4.3 V3 — WiFi Toggle

Builds on V1 + V2; introduces a shared-state risk that wasn't visible until integration.

Requirements added: REQ-3: LED shall be toggle-able via a WiFi HTTP request. REQ-4 (added mid-cycle): LED state shall remain consistent regardless of which input last changed it.

Design delta: Minimal HTTP server added; handleToggle() writes to the same shared ledState variable that the timer and button paths use. Architecture note added: three writers now share one piece of state.

Code delta: WiFi setup in setup(), handleToggle() endpoint, ledState changed to a variable guarded against concurrent writes.

Unit testing: TC-3.1: verify the HTTP endpoint returns the expected response independent of the LED hardware.

Integration testing: TC-3.2: confirm an HTTP request from a phone/laptop actually drives the physical GPIO and LED.

System / Regression testing: TC-3.3: re-run V1's timer test and V2's button test with WiFi active, to confirm neither regressed. This is where the race condition was caught — rapid alternating button/WiFi toggles occasionally left the LED in the wrong state.

Acceptance testing: TC-4.1/TC-4.2: rapid alternating inputs from all three sources produce a consistent, glitch-free final LED state, satisfying REQ-4 as well as REQ-1–REQ-3.

Deliverable: Updated sketch (esp32_led_v3.ino) adding the HTTP toggle endpoint and shared-state guard, an updated architecture note on the three writers to ledState, and the full regression + race-condition test log satisfying REQ-3 and REQ-4.

# 5. Takeaway

- Keep the change log and traceability matrix updated after every cycle, not just at the end.

- Every new cycle re-runs the previous cycles' key tests as regression checks — don't assume old features stay correct.

- New requirements (like REQ-4) can be added mid-project as integration reveals gaps; just give them the same design/code/test trail as any other requirement.

- The matrix, not the code, is the source of truth for “is this project done” — done means every row has a passing test.