ESP32 LED Control

Proof-of-Concept Project Plan — V-Model Approach

# 1. Project Overview

Goal: get hands-on with the ESP32 by building a small, complete project rather than a single script. Controlling an LED is the classic “Hello World” of embedded development, but running it through the full V-Model turns it into a rehearsal for how real embedded projects are planned, built, and verified — the process matters as much as the blink.

Suggested scope, in increasing difficulty (pick one or progress through all three):

- Stage 1 — Blink: LED turns on/off automatically on a timed loop.

- Stage 2 — Input control: a push-button (or the ESP32's BOOT button) toggles the LED.

- Stage 3 — Remote control (stretch goal): LED is toggled over Wi-Fi via a tiny web page or serial command.

The plan below is written generically so it covers whichever stage you choose — just substitute your chosen input method wherever “control input” appears.

# 2. What the V-Model Is (and why bother for a blink)

The V-Model pairs each development phase on the left with a corresponding test phase on the right, forming a V. Every design decision has a matching verification step, so nothing gets “built then forgotten.” For a small project this is overkill in a corporate-process sense, but it's a great way to practice thinking like an embedded engineer: define what “done” means before you write code, then prove you hit it.

| Development Phase (left side) | Verification Phase (right side) | Purpose |
| --- | --- | --- |
| Requirements Analysis | Acceptance Testing | Confirm the finished project actually does what you set out to build |
| System Design | System Testing | Confirm the whole board + circuit + code works together end-to-end |
| Architecture Design | Integration Testing | Confirm hardware and firmware talk to each other correctly |
| Module (Detailed) Design | Unit Testing | Confirm each individual function behaves correctly in isolation |

The bottom of the V, where the two sides meet, is Coding — where design becomes firmware.

# 3. Phase-by-Phase Plan

## 3.1 Requirements Analysis

Define what the project must do before touching hardware.

- Functional: “The system shall turn the LED on/off in response to [control input].”

- Functional: response to the control input shall be visible within ~200 ms.

- Non-functional: run from USB power (5V); LED current shall stay within the GPIO's safe limit (~12 mA recommended, 40 mA absolute max on most ESP32 pins).

- Constraint: use parts on hand or easily available (ESP32 dev board, LED, resistor, breadboard, jumper wires).

Deliverable: a short written spec (even 5 bullet points in a notes file) — this is what Acceptance Testing will check against later.

## 3.2 System Design

Decide the overall shape of the solution.

- Choose the control input for this stage (timer / button / Wi-Fi).

- Choose an LED GPIO pin (e.g., GPIO2, often tied to the onboard LED) and, if used, a button pin (e.g., GPIO0 / BOOT button).

- Sketch a simple state diagram: OFF → (trigger) → ON → (trigger) → OFF.

Deliverable: one-page block diagram — board, LED, button/Wi-Fi, power source — and the state diagram.

## 3.3 Architecture Design

Bridge system design into concrete hardware + software structure.

- Hardware: GPIO pin → current-limiting resistor (~220–330Ω) → LED anode; LED cathode → GND.

- Software: decide polling vs. interrupt for button input; decide whether Wi-Fi runs in the main loop or a FreeRTOS task.

- Define a pin map table (GPIO number → function) so wiring and code stay in sync.

Deliverable: circuit sketch/photo of breadboard layout, plus the pin map table.

## 3.4 Module (Detailed) Design

Design the individual functions before writing all of them at once.

- setup() — configure pin modes, initialize serial/Wi-Fi as needed.

- loop() — read input, decide LED state, call output function.

- ledOn() / ledOff() / toggleLed() — isolated GPIO write logic.

- readButton() — includes debounce logic (e.g., 20–50 ms) if using a physical button.

Deliverable: pseudocode or a simple function list with inputs/outputs for each.

## 3.5 Coding

Implement in Arduino IDE or PlatformIO (VS Code) using the ESP32 board package. Keep commits small — e.g., “blink works,” “button toggle works,” “Wi-Fi endpoint works” — so each commit maps to a design step above.

## 3.6 Unit Testing

Test each function from 3.4 in isolation, mostly via the Serial Monitor.

- Verify ledOn()/ledOff() actually change the physical LED state.

- Verify readButton() reports clean presses with no false triggers (debounce check).

- If Wi-Fi is used, verify the endpoint returns the expected response independent of the LED.

## 3.7 Integration Testing

Confirm hardware and firmware work together, not just in isolation.

- Flash the board and confirm GPIO commands actually toggle the physical LED on the breadboard.

- Confirm button presses (or Wi-Fi requests) reliably change LED state with the real circuit connected.

- Check current draw is within safe limits (multimeter, if available).

## 3.8 System Testing

Test the complete assembled system end-to-end under realistic conditions.

- Power-cycle the board and confirm it boots into the correct initial LED state.

- Run repeated toggle cycles (e.g., 50 presses) to check for missed triggers or hangs.

- If Wi-Fi is used, test from a phone and a laptop on the same network.

## 3.9 Acceptance Testing

Check the finished project against the original requirements from 3.1 — the top of the V closing the loop.

- Does the LED respond to the chosen control input as specified?

- Is the response time under ~200 ms?

- Does it run safely on USB power within GPIO current limits?

If all boxes check out, the proof of concept is complete and ready to extend to the next stage.

# 4. Bill of Materials

| Item | Qty | Notes |
| --- | --- | --- |
| ESP32 dev board | 1 | Any common variant (e.g., ESP32-DevKitC, NodeMCU-32S) |
| LED | 1 | Any color; onboard LED (often GPIO2) works too if no external LED on hand |
| Resistor | 1 | 220–330 Ω, current-limiting for the LED |
| Breadboard | 1 | Half-size is plenty |
| Jumper wires | 2–3 | Male-to-male |
| Push-button (optional) | 1 | Needed for Stage 2 (input control) |
| USB cable | 1 | Data-capable, matching the board's port |
| Arduino IDE or PlatformIO | — | With ESP32 board support installed |

# 5. Suggested Timeline

| Phase | Estimated Time | Milestone |
| --- | --- | --- |
| Requirements + System Design | 30–45 min | Spec + block diagram written |
| Architecture + Module Design | 30–45 min | Circuit sketch + pin map + pseudocode done |
| Coding (Stage 1: Blink) | 30–60 min | LED blinks on a timer |
| Unit + Integration Testing | 20–30 min | GPIO + debounce verified |
| System + Acceptance Testing | 20–30 min | Full power-cycle + requirement checklist passes |
| Stage 2 / Stage 3 (optional) | 1–2 hrs | Button or Wi-Fi control added, same V repeated |

# 6. Acceptance Checklist

- [ ] LED responds correctly to the chosen control input

- [ ] Response time is under ~200 ms

- [ ] Board boots to the correct default LED state after power loss

- [ ] No excessive current draw or hardware warmth

- [ ] Code is committed in small, phase-aligned steps